# How to install new logos

1: Unzip the archive

2: Using Altair, locate the directory where you install UK Timers (probably Auto Start).

3: There you will find a sub-directory there called “UK TAP Project”, navigate to that.

4: Copy the logo.dat file to your Toppy. Put it in the “UK TAP Project” folder.

If you can’t find the “UK TAP Project” folder, you probably didn’t copy it across during the original install. No problem, just copy across now.

# Logos for other countries

We think we’ve included all the logos for current UK channel line up on Freeview. Obviously that’s not much use for those users living around the world.

Kidhazy, tyreless, and ando9185 has done a great job of producing logos for Australian users. http://chris-anderson.customer.netspace.net.au/

# We would like to hear from you if you produce any logos for your country.

# Making your own logos

You may want to customise the channel logos, or add new ones. Here’s how to do it.

1: Create a gif image of the logo of 38 x 60 pixels. 
2: Copy all your logos to the “SourceLogos” folder on your Toppy. It’s located under the “UK TAP Project” folder, which is itself under folder where you installed UK Timers.

4: From the Toppy, run the “MakeLogos” TAP in the “UK TAP Project” folder. 
5: You can either preview the gif files, the dat file or create a dat file. 
6: Create a dat file.

7: Your new logos will load the next time your Toppy comes out of standby.

Note: The logo filenames must match the Toppy channel numbers, unlike Improbox, these names ARE case sensitive and should have all punctuation (excluding spaces) removed from their names.

We’ve included our source logos in this archive, so by all means alter them to suit your needs. There you will also find the “MakeLogos” TAP which you will need to generate the logo.dat file.

38x60 pixels is the ideal size; you can use a smaller size if you like, but they will look a little odd.